Fiche 12

Le port et la sécurité du port

Fiche 12.3 dragages et sondages, effectués par divers types d’engins.

Drague désagrégatrice DARTAGNAN.

L'élinde en forme de poutre massive d'un poids de 1300 tonnes porte un désagrégateur entraîné par moteur électrique.

La mixture est pompée par trois pompes centrifuges(une dans la poutre deux sur la drague), elles peuvent refouler dans un collecteur de 1 m de diamètre sur une distance de 1 km.

La navire est maintenu à poste par un pieu principal et deux de 200 tonnes installés à l'arrière. le système permet le rangement à l'horizontal des pieux et comprend un système d'amortisseur de houle par câbles réas et vérins hydrauliques.

La drague est équipée d'une grue se déplaçant sur rail et sur toute la longueur du navire.

Deux potences permettent la mouillage de manière autonome des deux ancres qui serviront aux déhalages successifs en manœuvre de dragages en demi cercle (papillonnage).

L:123.81	Lpp:104.40	L:25.20		Creux:8.20	TE:6.15

Profondeur de dragage:6/35

Vitesse 12.5nds.

Equipage: 35

DP/DT trailing suction hopper dredger

Drague aspiratrice Equipée de moyens indépendants de manutention

Antigoon Construction year:   1994/2002

Construction year: 1989 Dimensions: Length o.a.:115.00 m Breadth o.a.:22.46 m Draught max.:8.68 m

Moulded depth (creux sur quille):9.80 m

Dredging depth: 45.00 m

Suction pipe Ø: 1,200 mm

Discharge pipe Ø: 900 mm

Hopper capacity: 8,400 m3

Loading capacity: 12,700 t

Max. speed loaded: 14.00 knots

Power:

Total installed:		9,735 kW

On pumps: 		ps (dredging) 4,000 kW

shore delivery 		ps+sb 6,500 kW

Propulsion: 	sailing 8,000 kW

trailing 4,100 kW

DP/DT trailing suction hopper dredger

Pearl River(*)

Dimensions: Length o.a.:182.22 m Breadth o.a.:28.00 m Moulded depth:11.90 m Draught max.:10.77 m

Dredging depth :   30/50/60/120 m Suction pipe Ø:   2 x 1,200 mm Discharge pipe Ø:    1,000 mm

Hopper capacity:   24,146 m³ Loading capacity:   35,025 t

Max. speed loaded:   15.00 knots

Bucket dredger

Volkracht II

Construction year: 1967

Dimensions: Length o.a.:54,10 m  Breadth o.a.:9,65 m Draught max.:2,10 m

Moulded depth:3,32 m

Dredging depth max.: 12/16 m

Capacity (buckets) 630/730/900 l

Total installed power 503 kW

Backhoe dredger

Pinocchio

Construction year:  1999/2005

Dimensions: Length o.a.:60.00 m Breadth o.a.:19.00 m Draught max.:2.50 m

Moulded depth:4.50 m

Spuds:		 3

Length:	 45.00 m

Aqua Digger Lebherr P996:   	5 m

Bucket: 			5.7/4.5/3.0/10.0/24.0 m³

Monoboom 			25.0/18.5/14.0 m

Stick: 				9.6/12.0 m

Max. digging depth  		38.1 m

Power: Total installed: 2 x 1,103  kW

DP/DT operated spreader pontoon

Bayard II

Construction year: 1997/2002

Dimensions: Length o.a.:54.16 m  Breadth o.a.:21.24 m Draught max.:1.60 m

Moulded depth:		2.24 m

Spuds:			2 x Ø 900 mm

Length: 		23.00 m

Discharge pipe Ø: 	850 mm

Loading capacity: 	1,050 t

Total installed power: 	696kW

D.P. Fall Pipe Vessel

Rollingstone

Dimensions: Length o.a.: 139.00 m Breadth o.a.:32.00 m Draught Loaded:  6.60 m

Loading Capacity:  	12,000 ton

Main Propellors   	2 x 3,125 kW

Azimuth Thrusters   	2 x 650 kW (retractable) Bow Thruster   	2 x 300 kW (tunnel)

Cruising Speed   	12 kts

Dynamic Positioning

Fall Pipe

Steel pipe sections (8 m)  diameter1,000 / 500 mm  depth up to 1,000 m

Special Service Stone Dumping Vessel IMO / ISM-Code

DP side stone dumping vessel

Pompeï ** Joint ownership

Construction year:   1988

Dimensions: Length o.a.:65.50 m  Breadth o.a.:16.03 m  Draught max.:3.57 m

Carrying capacity:   	1,809 t

Load:    		30t/m 2

Discharging time:    	11 min

Mooring system:	8 x 500 m

Dynamic positioning:

Ocean going heavy lift catamaran

Rambiz

Construction year: 1986/2002

Lifting capacity 	4.000 tons (2x2.000 tons) 		68 metre mast

3.256 tons (1.676 + 1.580 tons) 	82 metre mast

2.000 tons

Loading capacity on deck 2.000 tons

Maximum speed 	8.5 knots

Length 85.00 metres  Beam 44.00 metres

Beam floaters: 19.90 metres

Depth 5.60 metres

Draught loaded 	3.70 metres

Draught sailing  	2.60 metres

Bunker capacity 	404 m3

Anchor system 8-point mooring, 4 double drum winches, 70 tons capacity

Accommodation crew 12, containerised accommodation for clients on request

Self-elevating platform

Nele

Type: Flexifloat S-50 Jack-up

Classification: BV - I 3/3 self-elevating platform

Length: 15.30 m  Width: 12.20 m  Depth: 1.52 m

Leg length: 24 m  Useful leg length: 20 m  Leg diameter: 30”

Payload: 		50 tonnes

Max. deckload: 		4 tonnes/m2

4 spuds